### Overall Merit

Strong Accept

### Summary

This paper addresses the memory wall problem in workloads dominated by long chains of dependent, indirect memory accesses. Existing techniques such as out-of-order execution and Precise Runahead Execution (PRE) fail to prefetch beyond the first few loads in these chains due to front-end bandwidth and back-end resource limitations.

The key idea is Vector Runahead, which dynamically vectorizes the runahead instruction stream rather than the original program. By speculatively executing multiple loop iterations as vector operations and extending the runahead termination condition, the processor can prefetch entire dependent load chains and generate much higher memory-level parallelism. Evaluation shows substantial performance improvements over both baseline OoO execution and PRE, with modest hardware overhead.

### Strengths

Identifies a fundamental limitation of existing runahead techniques

Elegant use of vectorization to amplify MLP

Strong performance gains on realistic, memory-latency-bound workloads

Low additional hardware complexity

### Weaknesses

Relies on wide SIMD support, limiting general applicability

Simplified handling of control-flow divergence

No evaluation of energy or power impact

### Comments

This is a clean and well-reasoned architectural paper that reframes runahead execution as a bandwidth and reordering problem. The idea is conceptually simple but powerful, and the evaluation convincingly demonstrates why vectorization is the right abstraction for prefetching dependent load chains.

### Discussion Points

Is Vector Runahead a general latency-hiding mechanism or primarily a niche optimization?