# 252A Paper Review Template

## Overall Merit

Strong Accept (4)

## Summary

Scalar computer performance is limited by the one instruction per cycle rate of issuing as well as by increasing memory communication time. To address these challenges, this paper presents a new type of architecture called decoupled access/execute (DAE). DAE separates memory access and execution to effectively increase the issue rate and hide latencies caused by memory accesses. The architecture is composed of an access processor and an execute processor that communicate with each other through queues. Each processor runs a separate program. The DAE architecture is compared against a CRAY-1 scalar architecture on 14 hand compiled Lawrence Livermore Loops workloads and achieves an average 1.71x speedup.

## Strengths / Weaknesses

Strengths

- Presents a novel architecture that achieves significant performance improvements

- Clear explanation of implementation details and addresses implementation challenges

Weaknesses

- Lacks discussion on PPA tradeoffs

- Lacks detailed performance analysis

## Detailed Comments

This paper presents a simple but elegant architecture. The design of the access process is reminiscent of load/store queues in modern OoO processors. DAE works well when the memory address is not data-dependent, so the accessing stream can run ahead of the executing stream. The accessing stream acts like a 100% accurate prefetcher in this case, similar to runahead execution. It’s interesting to see how although OoO execution has surpassed DAE, many concepts of DAE have continued to live on.

The core benefit of DAE architectures is that they hide the long latencies of memory operations. I would emphasize that more in the introduction than the one instruction per clock period limitation.

What are the overheads of DAE? For instance, DAE has a higher program size overhead since each branch instruction on CRAY-1 becomes 2 branch instructions on DAE architectures.

Plus, in workloads where the DAE is reduced to a serial machine, what is its overhead compared to a scalar machine? Certain types of workloads might reduce DAE architectures to a serial machine and cause worse performance, i.e. pointer chasing where address calculation depends on data and the access processor must wait for the WAQ to empty before performing the address formation. I would also like to see a detailed performance analysis on the types of workloads that the DAE architecture performed better or worse on. For instance, I’d expect a lower speedup on arithmetic-heavy workloads.

The authors briefly mentioned out-of-order issue in the introduction. I would like to see a comparison of performance and area overheads between DAE and OoO issue.

DAE architectures seem to push a lot of complexity to the programmer, and introduce challenges like deadlock that programmers previously did not have to think about in a single machine setting.

## Discussion Questions

- What ideas of DAE architectures are still prevalent today?