Overall merit:

Accept

Summary:

To circumvent the bottleneck of single issue processors and high memory latencies, this paper proposes a processor with two concurrently executing instruction streams, one for memory accesses and one for other program execution. The two separate processors are connected by architecturally accessible FIFO queues which allow data passing between the two processors. Each processor fetches, decodes, executes, and writes back instructions as any normal processor does. The processors contain their own register files. The theoretical performance of the architecture is analyzed on a small set of loops. It shows the potential for substantial speedup over a traditional architecture.

Strengths:

Significant theoretical speedup over traditional processor.

Clever strategy for reducing the problem of memory access taking significantly longer than execution of non-memory instructions (decoupling them). Elegant, highly distinct microarchitectural techniques.

Weaknesses:

I think relying on the programmer to insert interlocks to avoid load after store hazards is a weakness. The compiler would have to protect all loads, and since the addresses are not known at compile time, this protection would need to be very conservative and could get ugly quickly.

No compiler, compiling done by hand.

Old school evaluation of designs was not what it is today.

No evaluation of area/power (would have been much harder back then).

Detailed Comments:

Thank you for submitting this paper to the 252A paper reading seminar. Overall, this is an interesting architectural solution to an important pair of bottlenecks for computer processors.

I think the proposed changes to the architecture are quite elegant, though I am curious to know how much extra hardware they require. Ie can each processor be significantly smaller than a single processor that does both, or is this more likely to double the hardware area or worse (it seems like having the queues and two separate register files will be significant area costs).

I think a compiler which reasons about deadlocks and which properly handles interlocking for load after store would be a necessity for any future work on this. Without that, this kind of processor will of course never be usable, and it is not clear how much more difficult it would be to compile for this architecture while guaranteeing not to have those issues.

Clarification question: what data is actually passed using the AEQ and EAQ, and is it easy to reason about what will be at the head of the queue at any moment in time? If not how do you deal with this?

Would having separate L1I and L1D caches solve the problem of having to interleave A and E instructions (to avoid a dual-ported memory)?