Paper: DECOUPLED VECTOR RUNAHEAD (Ajeya Naithani, Jaime Roelandts, Sam Ainsworth, Timothy M. Jones, Lieven Eeckhout, MICRO 2023)

Overall Merit: Strong Accept

Summary:

Traditional runahead techniques such as Vector Runahead (VR) require the Reorder-Buffer (ROB) to be full, and stall before speculative execution and memory-level parallelism occur. However, modern ROBs are huge, and rarely fill up. This causes the VR not to trigger often enough, introducing many stalled cycles during execution.

Decoupled Vector Runahead, contains an in-order runahead process that is decoupled from the main execution thread, thereby allowing it to speculate on future striding loads a lot sooner, and enable early prefetching. This hides memory latency, and greatly improves Memory-Level parallelism (MLP).

Evaluation across HPC workloads shows that DVR significantly increases MLP and achieves 2.4× average performance improvement over a baseline OoO processor with minimal hardware cost.

Strengths:

- Identifies pointer-chasing workloads as a major bottleneck in OoO computing

- Decouples runahead from ROB stalls

- Low hardware cost and overhead

- Handles nested loops well

- Strong evaluation with relevant benchmarks

Weaknesses:

- Complex microarchitecture to understand and implement

- Will make verification a harder task

- Would not work well on programs with little loop structure

Comments:

This is a very well-written technical paper that clearly describes the microarchitectural features of DVR. It also evaluates DVR across many workloads, and compares it with existing prefetchers, as well as the ideal scenario: Oracle. This makes the results section very strong, and proves that DVR outperforms VR on all benchmarks.

This paper would require me to read over it multiple times to fully understand the microarchitectural features as it is very dense and information heavy. Like all microarchitectural papers I have read so far, it is challenging to follow when the features are described in words, rather than diagrams, especially for someone new in computer architecture. I am a big advocate for diagrams. And I believe that diagrams are worth a thousand words.

If I could rewrite this paper, I would include many more diagrams, one for every sub-block of DVR. This paper only contains one system diagram.

Questions:

- Even though the hardware overhead is small, how much additional energy does DVR consume compared to other prefetchers? It would be interesting to see a study on this.

- How does DVR perform on irregular access patterns?

Nits:

- I would include more diagrams to illustrate microarchitectural features.