# 252A Paper Review Template

## Overall Merit

Strong Accept (4)

## Summary

Computing CPI stacks for superscalar out-of-order processors is challenging because many stall-causing events may overlap. This work takes a top approach and presents a performance counter method to build accurate CPI stacks. The work first presents their performance model derived from interval analysis. The work defines a miss event as an event that eventually stops the issuing of useful instructions. A program execution can then be split into intervals where issuing of useful instructions is disrupted, and the causes for this disruption can then be attributed to various frontend and backend misses. The work presents a counter architecture based on this, and evaluates their design on the SimpleScalar/Alpha v3.0 simulator. They compare their design against a simulator-based CPI stack, naive methodologies, and the IBM POWER5 mechanism.

## Strengths / Weaknesses

Strengths

- The paper presents an accurate method to derive CPI stacks from superscalar OoO processors

- The paper proposes a hardware implementation of these counters

Weaknesses

- Only addresses a limited set of miss types

## Detailed Comments

Overall, I thought this was a well-written paper. I thought this paper presented a very clear explanation of their performance model and interval analysis method. I appreciated their discussion on the counter architecture implementation, which I think demonstrated enough the feasibility of their design.

I did not fully follow the paper’s critique of the IBM POWER5 method. I thought that it made sense for the IBM POWER5 to only count miss events when the ROB was either stalled or empty, since only those cases directly translate to a decrease in CPI. But the paper consistently critiqued POWER5 for only counting during those ROB events.

Since the paper assumes a well-balanced processor design, it doesn’t account for some stalls. For instance, in a processor with some complex instructions that require complex decode, this work would not account for stalls caused by decode inefficiency. Because of this, this work is probably more insightful for software-level optimizations than processor architecture design.

## Discussion Questions

- Would this methodology be useful in modern complex OoO cores?